| Python Code: | Annotations: |
| --- | --- |
| import pandas as pd import numpy as np import matplotlib.pyplot as plt import seaborn as sns | Imports all necessary packages into Python to use |
| sns.set_style('whitegrid') sns.set_palette('deep') | Sets the styles and palettes for charts |
| nhts_data = pd.read_csv('NHTS.csv') ngsim_data = pd.read_csv('NGSIM.csv') | Uses pandas to read in both CSV files |
| fuel_counts = nhts_data['fuel_type'].value_counts().dropna() | Creates a variable for fuel counts using the NHTS data category “fuel type” and obtaining the value counts |
| plt.figure(figsize = (12,6)) | Plots an empty figure with size 12x6 |
| fuel_counts.plot(kind = 'bar', color = sns.color_palette('viridis',len(fuel_counts))) | Creates a bar chart of the fuel counts data |
| plt.xlabel('Fuel Type') plt.ylabel('Number of Vehicles') plt.title('Vehicle Number by Fuel Type', fontsize = 15) | Sets axis labels and title, with a font size 15 |
| plt.xticks() plt.legend(fontsize = 12) plt.show() | Creates the legend and shows the bar chart |
| plt.figure(figsize = (10,5)) | Plots an empty figure with size 10x5 |
| sns.histplot(ngsim_data['leader_acc(m/s^2)'], bins = 30, kde = True, label = 'Leader Acceleration', color = 'steelblue', alpha = 0.6) sns.histplot(ngsim_data['follower_acc(m/s^2)'], bins = 30, kde = True, label = 'Follower Acceleration', color = 'coral', alpha = 0.6) | Sorts the leader and follower accelerations into bins of size 30, includes different colors |
| plt.xlabel('Acceleration (m/s^2)', fontsize = 12) plt.ylabel('Frequency', fontsize = 12) plt.title('Distribution of Acceleration for Leader and Follower Vehicles', fontsize = 14) | Sets the axis labels and title |
| plt.legend(fontsize = 11) plt.show() | Creates legend and shows plot |
| plt.figure(figsize = (10,5)) | Plots an empty figure with size 10x5 |
| plt.figure(figsize = (10,5)) boxplot = nhts_data.boxplot(column = 'vehicle_age', by = 'make') | Sets a boxplot variable equal to a boxplot sorted by vehicle age and make |
| plt.xticks(rotation = 90) plt.xlabel('Make', fontsize = 12) plt.ylabel('Vehicle Age (years)', fontsize = 12) plt.title('Boxplot of Vehicle Age by Make', fontsize = 14) | Sets the axis labels and title |
| plt.show() plt.savefig("my_boxplot.png", dpi = 300, bbox_inches="tight") | Shows the plot and makes the format “tight” |
| trajectory_number = 1 data_subset = ngsim_data[ngsim_data['trajectory_number'] == trajectory_number] | Selects a trajectory number and creates a variable based on it |
| gap = data_subset['leader_position(m)'].values - data_subset['follower_position(m)'].values | Creates a variable for the gap in distance between the leader and follower vehicles |
| time = data_subset['Time'].values | Creates a variable for time |
| fig, axes = plt.subplots(2, 2, figsize=(14, 10)) | Creates a figure with subplots |
| axes[0,0].plot(time, data_subset['leader_position(m)'].values, 'b-', label = 'Leader', linewidth = 1.5) axes[0,0].plot(time, data_subset['follower_position(m)'].values, 'r-', label = 'Follower', linewidth = 1.5) | Creates a figure in the top left, and includes the leader and follower position values with respect to time |
| axes[0,0].set_ylabel('Position (m)', fontsize = 11) axes[0,0].set_title('Position vs Time', fontsize = 11) | Sets a y label and title |
| axes[0,0].legend(fontsize = 10) axes[0,0].grid(True, alpha = 0.3) | Creates a legend for the chart |
| axes[0,1].plot(time, data_subset['leader_speed(m/s)'].values, 'b-', label = 'Leader', linewidth = 1.5) axes[0,1].plot(time, data_subset['follower_speed(m/s)'].values, 'r-', label = 'Follower', linewidth = 1.5) | Uses the same method as above, however this chart is in the top right and uses the leader and follower vehicle speeds |
| axes[0,1].set_ylabel('Speed (m)', fontsize = 11) axes[0,1].set_title('Speed vs Time', fontsize = 11) | Sets a y label and title |
| axes[0,1].legend(fontsize = 10) axes[0,1].grid(True, alpha = 0.3) | Creates a legend for the chart |
| axes[1,0].plot(time, data_subset['leader_acc(m/s^2)'].values, 'b-', label = 'Leader', linewidth = 1.5) axes[1,0].plot(time, data_subset['follower_acc(m/s^2)'].values, 'r-', label = 'Follower', linewidth = 1.5) | Uses the same method as above, however this chart is in the bottom left and uses the leader and follower vehicle accelerations |
| axes[1,0].set_ylabel('Acceleration (m/s^2)', fontsize = 11) axes[1,0].set_title('Acceleration vs Time', fontsize = 11) | Sets a y label and title |
| axes[1,0].legend(fontsize = 10) axes[1,0].grid(True, alpha = 0.3) | Creates a legend for the chart |
| plt.suptitle(f'Vehicle Pair Dashboard - Trajectory {trajectory_number}', fontsize = 16) plt.tight_layout() plt.show() | Creates a title for all four charts, sets the layout to tight, and shows the plots |
| def idm_acceleration(v, s, delta_v, v0, s0, T, a, b, delta): | Defines a function for idm acceleration, taking eight variables |
| s_star = s0 + v * T + (v * delta_v) / (2 * np.sqrt(a * b)) s_star = max(s_star, s0) | Creates variable s_star and uses IDM equation to solve for it |
| acceleration = a * (1 - (v/v0)**delta - (s_star/s)**2) | Solves for acceleration using variables inputted into the function and the result of s_star |
| return acceleration |  |
| trajectory_number = 1 data_subset = ngsim_data[ngsim_data['trajectory_number'] == trajectory_number].reset_index(drop=True) | Sets the trajectory number to one, and sets a variable to a subset of data at trajectory one |
| time_data = data_subset['Time'].values | Sets a variable for the the time data in the dataset |
| leader_position = data_subset['leader_position(m)'].values leader_speed = data_subset['leader_speed(m/s)'].values | Sets variables for the leader position and speed |
| dt = 0.1 | Creates and sets variable dt to 0.1 |
| v0 = 30 s0 = 2 T = 1.5 a_param = 1.0 b = 1.5 delta = 4 | Sets the initial conditions for the variables that will be inputted into the idm acceleration function |
| n_steps = len(time_data) sim_position = np.zeros(n_steps) sim_speed = np.zeros(n_steps) sim_acc = np.zeros(n_steps) | Set variables for the sim position, sim speed, and sim account |
| sim_position[0] = data_subset['follower_position(m)'].values[0] sim_speed[0] = data_subset['follower_speed(m/s)'].values[0] | Sets initial conditions for sim position and sim speed |
| for i in range(n_steps - 1): | Creates a for loop to iterate through i |
| gap = max(leader_position[i] - sim_position[i], 0.1) delta_v = sim_speed[i] - leader_speed[i] | Adjusts the gap and delta variables based on the changing sim position |
| sim_acc[i] = idm_acceleration(sim_speed[i], gap, delta_v, v0, s0, T, a_param, b, delta) sim_speed[i+1] = max(sim_speed[i] + sim_acc[i]*dt, 0) sim_position[i+1] = sim_position[i] + sim_speed[i] * dt | Updates the sim position as the function iterates through i |
| plt.figure(figsize = (12,6)) |  |
| plt.plot(time_data, leader_position, 'k--', linewidth = 2, label = 'Leader (NGSIM)') plt.plot(time_data, data_subset['follower_position(m)'].values, 'b-', linewidth = 2, label = 'Real Follower (NGSIM)') plt.plot(time_data, sim_position, 'r-', linewidth = 2, label = 'Simulated Follower (IDM') | Pots the leader, follower, and simulated follower positions |
| plt.xlabel('Time (seconds)') plt.ylabel('Position (m)') | Includes x and y axis titles |
| plt.legend() plt.grid(True, alpha=0.5) plt.show() | Plots the legend and shows the figure |